SECTION 10450

PEDESTRIAN CONTROL DEVICES

For best results, display hidden notes to specifier.

** NOTE TO SPECIFIER ** Turnstile Security Systems Inc.; Turnstiles, 3 Arm Turnstiles, Gates, Posts and Rails, Portable Post Systems, Shopping Cart Control.

.

This section is based on the products of Turnstile Security Systems Inc., which is located at:

245 Matheson Blvd. East, Unit 14

Mississauga, Ontario L4Z 3C9

Tel: (905) 890-2222

Fax: (905) 890-3786

Toll Free: 1 (888) 371-2222

E-mail: info@turnstilesecurity.com

Web Site: www.turnstilesecurity.com

.

Turnstile Security Systems Inc. offers a complete line of turnstiles, 3 arm turnstiles, hi-gate turnstiles, gates, posts and rails, portable post systems, shopping cart control. Turnstile Security Systems Inc. engineers have exceeded user expectations for reliable, safe and functional gates. The basic designs have established an enviable record of service over the past 20 years.

.

SECTION 10450 -  PEDESTRIAN CONTROL DEVICES, Copyright 2003, ARCAT, Inc.

- GENERAL

  - SECTION INCLUDES

    - 4 Arm Turnstiles.

    - 3 Arm Turnstiles.

    - Hi-Gate Turnstiles

    - Swing Gates.

    - Posts and Rails.

    - Portable Post Systems.

    - Shopping Cart Control.

  - REFERENCES

    - ADAAG - Americans with Disabilities Act Accessibility Guidelines; U.S. Architectural and Transportation Barriers Compliance Board.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - [ Product Data ]:  Manufacturer's descriptive literature for equipment specified, including components and accessories.

    - Shop Drawings:  Indicate location of each individual pedestrian control device in the project.

** NOTE TO SPECIFIER **  Delete the following paragraph if specifying colors in this section.

    - Selection Samples:  Two sets of color chips representing manufacturer's full range of available colors and finishes for each device specified.

    - Verification Samples:  Two samples, minimum size 6 inches, for each color or finish specified.

    - Manufacturer's instructions:  Printed installation instructions for each product.

  - QUALITY ASSURANCE

    - Regulatory Requirements:  Sign types to comply with ADAAG requirements.

  - DELIVERY, STORAGE, AND HANDLING

    - Store products of this section in manufacturer's unopened packaging until installation.

    - Maintain dry, heated storage area for products of this section until installation of products.

  - WARRANTY

    - All Turnstile Security System's products are warranted against defects in material or workmanship for a period of 18 months from date of shipping.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: Turnstile Security Systems Inc: 40 Erin Park Drive, Erin, Ontario N0B 1T0.  ASD. Tel: (519) 833-9494. Fax: (519) 833-9495. Toll Free: 1 (888) 371-2222. Toll-Free Fax: (888) 371-7922. E-mail: info@turnstilesecurity.com . Web Site: www.turnstilesecurity.com .

** NOTE TO SPECIFIER **  Delete one of the two following paragraphs; coordinate with Division 01 requirements.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600.

    - Substitutions:  Not permitted.

** NOTE TO SPECIFIER **  Delete following paragraph if substitutions not permitted.

    - Unless otherwise specified for an individual product or material, supply all products specified in this section from the same manufacturer.

  - EQUIPMENT

** NOTE TO SPECIFIER **  Delete products not required; consult manufacturer's catalog for equipment configuration and available color and size options.

    - 4 Arm Turnstiles:. 4 Arm turnstiles are constructed with heavy-gauge materials and bearings. Parts are made of high-grade plated steel with smooth curved edges. Bolt heads are hidden by a cover which snaps over the floor mounting flange.

** NOTE TO SPECIFIER **  Select components required, delete items not required. Header post and curved rail section are required for complete installation. Handicapped gate may be required together with this device to permit wheelchair and stroller access.

      - Turnstile Cat. No. 1001.

      - Turnstile Cat. No. 1002.

      - Header Post Cat. No 1003

      - Curved Rail Section (2 Curves, 2 Posts) Cat. No. 1004

      - Safety Release header Post Cat. No. 1003SR releases in case of emergency.

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required.

      - Provide with fitting for adjacent rail as indicated.

** NOTE TO SPECIFIER **  Select required rotation, delete the rotation not required.

      - Rotation:

        - Clockwise.

        - Counterclockwise.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

    - 3 Arm Turnstiles: 3 arm turnstiles are constructed with heavy-gauge materials and bearings. Parts are made of high-grade steel. The hydraulically controlled mechanism smoothly repositions the arm to a closed position after each passage. The main indexing cam is  fabricated of Selrin for years of maintenance free service. The arm rotates easily and is engineered to be extremely quiet. All bearings are permanently lubricated. Internal parts are hardened for many years of reliable service. Oil impregnated bronze bushings are used for low maintenance.

** NOTE TO SPECIFIER **  Select components required, delete items not required. Handicapped gate may be required together with this device to permit wheelchair and stroller access.

      - Operation:

        - One way operation.

        - Two way operation.

      - Control:

        - Mechanical controlled. Cat. No. 1100.

        - Electrically controlled. Cat. No. 1100-24.

      - Options:

        - Electronic Counting.

        - Card reader Interface.

        - Coin or token interface.

        - Portable platform.

** NOTE TO SPECIFIER **  Select finish required delete finish not required.

      - Finish:

        - Stainless steel finish.

        - Stainless steel and powder coat finish. Color as selected.

    - Hi-Gate Turnstiles: 3 rotor construction, heavy -gauge materials and bearings. Parts are made of high grade steel. Hydraulically controlled mechanism smoothly repositions arms to a closed position after each passage. The arms rotate easily and is engineered to be extremly quiet. Main indexing cam is fabricated of Delrin for years of maintenance free service. Internal parts are hardened for many years of reliable service. Oil impregnated bronze bushings are used for low maintenance.

** NOTE TO SPECIFIER ** Select Operation, Control and Option requirements from the following paragraphs and delete requirements not required. Handicapped gate may be required together with this device to permit wheelchair and stroller access.

      - Operation:

        - One Way Operation

        - Two Way Operation

      - Control:

        - Mechanically Controlled. Cat. No. HGGM-3000.

        - Electrically Controlled. Cat. No. HGGE-3000.

      - Options:

        - Electronic counting

        - Card reader interface

        - Coin or token interface

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required.

      - Provide with fitting for adjacent rail as indicated.

** NOTE TO SPECIFIER **  Select required rotation, delete the rotation not required.

      - Rotation:

        - Clockwise.

        - Counterclockwise.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is galvanized.

      - Finish:

        - Galvanized.

        - Powder Coat. Color as selected.

        - Stainless steel.

    - Swing Gates Self Closing: Hydraulic arm closure with 90 degree swings in one direction and a unique breakaway safety feature to provide an exit path in case of emergency.

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Standard single swing gate. Cat. No. 2001, 2002.

        - Standard double swing gate. Cat. No. 2003, 2004.

        - Custom swing gate.

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required. Coordinate with Posts and Rails specified in this Section.

        - Provide with fitting for adjacent rail as indicated. Cat. No. 1016.

** NOTE TO SPECIFIER **  Select arm opening size required delete sizes not required. Consult with Turnstile Security Systems for custom sizes.

      - Arm Openings:

        - 29 inches (737 mm).

        - 33 inches (838 mm).

        - 36 inches( 914 mm).

        - Custom size _______.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

    - Swing Gates Electrically Locking/Unlocking: Hydraulic arm closure with 90 degree swings in one direction and a unique breakaway safety feature to provide an exit path in case of emergency.

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Standard single swing gate entrance. Cat. No. 2021, 2022.

        - Standard double swing gate. Cat. No. 2023, 2024.

        - Custom swing gate.

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required.

        - Provide with fitting for adjacent rail as indicated. Cat. No. 1016.

** NOTE TO SPECIFIER **  Select arm opening size required delete sizes not required. Consult with Turnstile Security Systems for custom sizes.

      - Arm Openings:

        - 29 inches (737 mm).

        - 33 inches (838 mm).

        - 36 inches( 914 mm).

        - Custom size _______.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

** NOTE TO SPECIFIER **  Select control option required delete options not required. Can be operated manually or by floor mat, photo eye or other devices. Can also be interfaced with electronic security systems.

      - Control Options: Control module operates on 24 volt powered by a 120 VAC receptacle.

        - Electronic Counting.

        - Electronic for detection.

        - Electronic card reader Interface.

        - Audible lock and unlock signal.

        - Remote control.

    - Swing Gates Manual Closing: Manual arm closure with 90 degree swings in both directions. Gates are unlocked manually by the customer or a staff member. Unique breakaway feature to provide an exit path in case of an emergency.

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Standard swing high gate. Cat. No. 2031, 2032, 2033.

        - Standard double swing gate.

        - Custom swing gate.

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required. Coordinate with Posts and Rails specified in this Section

        - Provide with fitting for adjacent rail as indicated. Cat. No. 1016.

** NOTE TO SPECIFIER **  Select arm opening size required delete sizes not required. Consult with Turnstile Security Systems for custom sizes.

      - Arm Openings:

        - 29 inches (737 mm).

        - 33 inches (838 mm).

        - 36 inches( 914 mm).

        - Custom size _______.

      - Options:

        - Key lock and panic release. Cat. No. 2031, 2032.

        - Breakaway feature. Cat. No. 2031, 2032.

        - Slide locking. Cat. No. 2033.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

    - Swing Gates Handicap Type: Manual arm closure with 90 degree swings in both directions.

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Standard single swing gate. Cat. No. 2033.

        - Standard double swing gate. Cat. No. 2033.

        - Custom swing gate.

** NOTE TO SPECIFIER **  Rail fittings are optional, delete if not required. Coordinate with Posts and Rails specified in this Section

        - Provide with fitting for adjacent rail as indicated. Cat. No. 1016.

** NOTE TO SPECIFIER **  Select arm opening size required delete sizes not required. Consult with Turnstile Security Systems for custom sizes.

      - Arm Openings:

        - 29 inches (737 mm).

        - 33 inches (838 mm).

        - 36 inches( 914 mm).

        - Custom size _______.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

** NOTE TO SPECIFIER **  Select control required delete control not required.

      - Operation:

        - Mechanical latch on moving arm.

        - Lockable with panic release.

        - Remote electrical control.

        - 507

** NOTE TO SPECIFIER **  Select Posts and Rails required delete those not required. Coordinate with Turnstiles gates and other equipment as required.

    - Posts and Rails: Railings and posts are modular. Provide in configuration indicated on the Drawings.

      - Posts. Posts are 2-1/4 inches (57 mm) O.D. 16 gauge steel with Bright Chrome plated steel.

** NOTE TO SPECIFIER **  Select posts and fittings for post and rail system required. Delete those not required.

        - Cat. No. 1011 Post (with no fittings).

        - Cat. No. 1013 Double Rail End Post (with two fittings).

        - Cat. No. 1014 Double Rail Center Post (with 4 fittings).

        - Cat. No. 1015 Double Rail Corner Post (with 4 fittings).

** NOTE TO SPECIFIER **  Select rail system required. Delete those not required.

      - Rails: Horizontal rails are 1-3/4 inch O.D. 16 gauge Bright Chrome plated steel and are attached to posts or walls with Turnstile Security Systems Inc.'s fittings.

        - 4 foot (1219 mm) pre cut for 48 inch (1219 mm) post centers - Cat. No. 1034

        - 5 foot 1524 mm) pre cut for 60 inch (1524 mm) post centers - Cat. No. 1035

        - 6 foot (1829 mm) - Cat. No. 1036

        - Custom lengths are made to order.

** NOTE TO SPECIFIER **  Select removable rail if required. Delete if not required. Indicate location on the Drawings.

      - Removable Rail: Cat. No. 1034-R, Cat. No. 1035-Ris designed to replace a standard #1034 or #1035 Horizontal Rail (45-1/4 inches (1149 mm) long). The finished length must be 1/4 inch (6 mm) longer than the space between the fittings.

      - Provide additional fittings as required by system selected.

    - Portable Post System:

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Classic.

        - Contemporary.

        - Executive.

        - Traditional

** NOTE TO SPECIFIER **  Select belting required delete those not required.

      - Belting: Nylon belting, 2 meters (6.56 feet) long that retracts firmly back into the head of the post.

        - Color as selected from the manufacturers standard colors.

        - Provide with printed Company Logo as directed.

        - Provide with printed Slogan as selected.

** NOTE TO SPECIFIER **  Select rope if required delete if not required.

      - Rope: Provide with hook or snap ends. Color shall be as selected from the manufacturers standard.

** NOTE TO SPECIFIER **  Select trim cap and stickers required delete those not required.

      - Trim Cap Stickers:

        - Provide to match post.

        - Provide stickers with custom logo as selected.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

        - Powder coat finish. Color as selected.

    - Portable Post System Sign Holders:

** NOTE TO SPECIFIER **  Select style required delete those not required.

      - Styles:

        - Classic.

        - Contemporary.

        - Executive.

        - Traditional

** NOTE TO SPECIFIER **  Select trim cap and stickers required delete those not required.

      - Trim Cap Stickers:

        - Provide to match post.

        - Provide stickers with custom logo as selected.

** NOTE TO SPECIFIER **  Select finish required delete finishes not required. Standard finish is bright chrome.

      - Finish:

        - Bright Chrome.

        - Satin Chrome.

        - Polished Brass.

        - Satin Brass.

        - Powder coat finish. Color as selected.

** NOTE TO SPECIFIER **  Select sign type required delete those not required.

      - Signs:

        - As selected from manufacturers standard signs.

        - Custom signs with messages as directed.

    - Shopping Cart Control: Fabricated of 2-1/4 inch (57 mm) OD, 16 gauge heavyweight chrome plated steel or galvanized for outside cart corral. Provided with chrome plated cover snaps over flange. Provide to configuration indicated on the Drawings.

    - Accessories:  Installation accessories specified in manufacturer's instructions.

- EXECUTION

  - EXAMINATION

    - Verify that surfaces to receive pedestrian control devices are properly prepared and ready to receive equipment.

    - Verify wiring and work provided by others is appropriate for the installation. Notify the Architect if conditions are unsatisfactory.

  - INSTALLATION

    - Install pedestrian control devices in accordance with manufacturer's instructions and the approved shop drawings. Comply with ADA requirements as required.

    - Prepare surfaces to assure a smooth, level and rigid surface.

    - Set fixed equipment with expansion bolts or other devices in accordance with the approved shop drawing.

    - Complete wiring  in accordance with the approved shop drawings.

** NOTE TO SPECIFIER **  If needed use the following Article together with the Drawings to identify and locate equipment project.  Delete if not required..

  - SCHEDULE

    - Equipment  Location  Finish   Options

END OF SECTION